SECTION 1 – PRODUCT AND COMPANY IDENTIFICATION

Product Name 	Tek Wash XL – Ultrasonic Cleaner

Updated Date	August 26, 2016

Containers	5 gallon, 20 gallon, 30 gallon, 55 gallon, 275 gallon

Company	Encore Industrial Products	Transportation Emergency (24 Hour) Contact

P.O. Box 300	 North America:	     1-877-703-3044 Ex. 405

Barker, TX 77413-0300

Phone Number 	(281) 944-4777

(877) 703-3044

Fax (281) 944-4778	      Medical Emergencies:

Contact your local Poison Control Center.

Transportation

DOT: CONSUMER COMMODITY, ORM-D

IATA: CONSUMER COMMODITY, ORM-D

IMDG: CONSUMER COMMODITY, ORM-D

SECTION 2 – HAZARDS IDENTIFICATION

Emergency Overview

Extremely flammable liquid and vapor; vapor may cause flash fires;

Hazard Classification: 3	Signal Word(s): Flammable  	  Hazard Symbol(s):

Hazard Statement: Fire, acute (immediate) health hazard, chronic (delayed) health hazard

Precautionary Statements:

- FLAMMABLE. Keep away from fire or flame.

- Do not use in eyes. In case of eye contact, flush thoroughly with water and seek medical attention.

- Stop use and ask doctor if irritation and redness develop and persists for more than 72 hours.

- Keep out of reach of children. If swallowed, get medical help or contact a Poison Control Center right away.

SECTION 3 – COMPOSITION / INFORMATION ON INGREDIENTS

Ingredients considered hazardous as defined by OSHA, 29 CFR 1910.1200, WHMIS under the HPA (Canada),

or ESIS (Europe).

Chemical Name	CAS Number		EINECS/ELINCS	Wt %	OSHA PEL	ACGIH TLV

Heptane		142-82-5

C7-C8 Alkanes		Mixture			60-100

C7-C8 Cycloalkanes		Mixture			7-13

Remaining ingredients are non-hazardous and/or present at amounts below reportable limits and considered confidential.

SECTION 4 – FIRST AID MEASURES

EYE CONTACT:		Flush eyes with cool, clean, low pressure water for at least 15 minutes. Hold eyelids apart to ensure complete irrigation of the eye and eyelid tissue. If easily accomplished, check for and remove contact lenses. If contact lenses cannot be removed, seek immediate medical attention. Do not use eye ointment. Seek medical attention.

SKIN CONTACT:		Remove contaminated shoes and clothing. Flush affected area with large amounts of water. If skin surface is damaged, apply a clean dressing and seek medical attention. Do not use ointments. If skin surface is no damaged, clean affected area thoroughly with mild soap and water. Seek medical attention if tissue appears damaged or if pain or irritation persists.

INHALATION:		Immediately move victim to fresh air. If victim is not breathing, immediately begin rescue breathing. If heart has stopped, immediately begin cardiopulmonary resuscitation (CPR). If breathing is difficult, 100 percent humidified oxygen should be administered by a qualified individual. Seek medical attention immediately.

INGESTION:		Do not induce vomiting. If spontaneous vomiting is about to occur, place victim’s head below knees. If victim is drowsy or unconscious, place on the left side with head down. Never give anything by mouth to a person who is not fully conscious. Do not leave victim unattended. Seek medical attention immediately.

SECTION 5 – FIRE FIGHTING MEASURES

FLASHPOINT (closed up):  			-9 deg C (15 deg F). (Tagliabue  (ASTM D-56) )

FLASHPOINT (Propellant):			Not determined

FLAMMABLE LIMITS:		       LEL: AP 1.0%		UEL: AP 6.7%

AUTOIGNITION TEMP: 		 	AP 246 deg C (475 deg F)

Hazardous Combustion Products:		Carbon dioxide, Carbon Monoxide, Smoke, Fumes, and/or unburned hydrocarbons.

FIRE EXTINGUISHING MEDIA: 	Small Fire: Use dry chemicals, carbon dioxide, foam, or inert gas (nitrogen). Carbon dioxide and inert gas can displace oxygen. Use caution when applying carbon dioxide or inert gas in confined spaces.

Large Fire: Use foam, water fog, or water spray, Water may be ineffective. Water may not extinguish the fire. Water fog and spray are effective in cooling containers and adjacent structures. However, water can be used to cool the external walls of vessels to prevent excessive pressure, auto ignition or explosion. Do not use solid stream of water directly on the fire as the water may spread the fire to a larger area.

SPECIAL INFORMATION: 	Flammable Liquid! This material releases vapors at or below ambient temperatures. When mixed with air in certain proportions and exposed to an ignition source, its vapor can cause a flash fire. Use only with adequate ventilation. Vapors are heavier than air and may travel long distances along the ground to an ignition source and flash back. A vapor and air mixture can create and explosion hazard in confined spaces such as sewers. If container is not properly cooled, it can rupture in the heat of a fire.

Protection of Fire Fighters:	 Firefighters must use full bunker gear including NIOSH-approved positive pressure self-contained breathing apparatus to protect against potential hazardous combustion or decomposition products and oxygen deficiencies. Evacuate area and fight the fire from a maximum distance or use unmanned hose holders or monitor nozzles. Cover pooling liquid with foam. Containers can build pressure if exposed to radiant heat; Cool adjacent containers with flooding quantities of water until well after the fire is out. Withdraw immediately from the area if there is a rising sound from a venting safety device or discoloration of vessels, tanks, or pipelines. Be aware that burning liquid will float on water. Notify appropriate authorities of potential fire and explosion hazard if liquid enters sewers or waterways.

SECTION 6 – ACCIDENTAL RELEASE MEASURES

Personal Precautions	Wear appropriate protective equipment (see Section 8).

Containment/Cleanup	Flammable Liquid! Release causes an immediate fire or explosion hazard. Evacuate all non-essential personnel from immediate area and establish a “regulated zone” with site control and security. A vapor-suppressing foam may be used to reduce vapors. Eliminate all ignition sources. All equipment used when handling this material must be grounded. Stop the leak if it can be done without risk. Do not touch or walk through spilled material. Remove spillage immediately from hard, smooth waling areas. Prevent spilled material from entering waterways, sewers, basements, or confined areas. Absorb or cover with dry earth, sand, or other non-combustible material and transfer to appropriate waste containers. Use clean, non-sparking tool to collect absorbed material.

SECTION 7 – HANDLING AND STORAGE

Handling	A spill or leak can cause an immediate fire or explosion hazard. Keep containers closed and do not handle or store near heat, sparks, or any other potential ignition sources. Avoid contact with oxidizing agents. Do NOT breathe vapor. Use only with adequate ventilation and personal protection. Never siphon by mouth. Avoid contact with eyes, skin, and clothing. Prevent contact with food and tobacco products. Do not take internally.

Non-equilibrium condition may increase the fire hazard associated with this product. A static electrical charge can accumulate when this material is flowing trough pips, nozzles or filers and when it is agitated. A static spark discharge can ignite accumulated vapors particularly during dry weather conditions. Always bong receiving containers to the fill pope before and during loading. Always confirm that receiving container is properly grounded. Always keep nozzle in contact with the container throughout the loading process.

Do not fill any portable container in or on a vehicle. Do Not use compressed air for filling, discharging or other handling operations. Product container is not designed for elevated pressure. Do NOT pressurize, cut, weld, braze solder, drill, or grind on containers. Do not expose product container to flames, sparks, heat, or other potential ignition sources. Empty containers may contain material residues which can ignite with explosive force. Observe label precautions

Storage		Keep container tightly closed. Store in a cool, dry, well-ventilated area. Store only in approved containers. Do not store with oxidizing agents. Do not store at elevated temperatures or in direct sunlight. Protect containers against physical damage. Head spaces in tanks and other containers may contain a mixture of air and vapor in the flammable range. Storage area must meet OSHA requirements and applicable codes

SECTION 8 – EXPOSURE CONTROLS / PERSONAL PROTECTION

Control Parameters  	Industrial Setting: Refer to Section 3 for ingredients with occupational exposure limits.

Engineering Controls	This is a consumer product that is safe for consumers under normal and reasonably

foreseeable use. For industrial settings, use in well ventilated area.

Personal Protective	Respiratory protection: Self-contained breathing apparatus if above TLV limit.

Equipment		Eye/Face: Safety glasses or goggles if splash hazards exist.

Skin: Chemical resistant gloves

Work Hygienic Practices	Always follow good hygienic work practices.

SECTION 9 – PHYSICAL AND CHEMICAL PROPERTIES

Color and Appearance	Clear liquid	VOC Content	NA

Odor			Solvent	Boiling Point	93 to 99oF

pH				Not Determined	Melting Point	Greater than 120oF

Specific Gravity		0.696	Vapor Density	Heavier than air

Flash Point		-9 deg C (15 deg F)	Solubility in Water	Negligible

SECTION 10 – STABILITY AND REACTIVITY

Chemical Stability		Stable

Conditions to Avoid		Keep away from heat, flame and other potential ignition sources; Keep away from strong oxidizing conditions and agents.

Incompatible Materials	Strong acids, alkalies, and oxidizers such as liquid chlorine and oxygen;

Hazardous Decomposition

Products		None

Hazardous Polymerization	Will not occur.

SECTION 11 – TOXICOLOGICAL INFORMATION

Toxicity Tests		Data is not available.

Potential Health Effects 	Acute Eye: Eye irritant. Will cause redness and burning sensation.

Acute Skin: Skin irritant. Will cause defatting of skin. Effects are reversible.

Acute Inhalation: Effects of overexposure include irritation of respiratory tract, headache, dizziness, nausea, and loss of coordination. Extreme overexposure may result in unconsciousness and possibly death.

Acute Ingestion: Aspiration hazard if swallowed.

Chronic Effects: Not expected to have chronic health effects.

Carcinogenicity: Not expected to be carcinogenic.

SECTION 12 – ECOLOGICAL INFORMATION

Ecological Tests: Data is not available.

Environmental Impact: The product ingredients are expected to be safe for the environment at concentrations predicted under normal use and accidental spill scenarios. Packaging components are compatible with the conventional solid waste management practices.

SECTION 13 – DISPOSAL CONSIDERATIONS

Disposal Methods	 Dispose of according to local, state, and federal regulations.

SECTION 14 – TRANSPORT INFORMATION

Identification	Proper		Hazardous	Packaging

Shipment	Number	Shipping Name		Classification      	Group

US DOT	UN1206	Heptane		3	II

IATA (air)	UN1206	Heptane			3	II

IMDG (vessel)	UN1206	Heptane		3	II

SECTION 15 – REGULATORY INFORMATION

SARA 311/312 Hazard Identification – This material would be classified under the following hazard categories:

Fire, Acute (immediate), Health Hazard, Chronic (delayed) Health hazard

SECTION 16 – OTHER INFORMATION

Legend

ACGIH	American Conference of Governmental 	ICL	In-Commerce List (Canada)

Industrial Hygienists	IMO	International Maritime Organization

CAA	Clean Air Act	N/A	Not Applicable

CAS	Chemical Abstract Service	NDSL	Non-Domestic Substances List (Canada)

CERCLA	CERCLA Hazardous Substances	N/E	Not Established

CFR	Code of Federal Regulations	NTP	National Toxicology Program

DOT	Department of Transportation	OSHA	Occupational Safety & Health Administration

DSL	Domestic Substances List (Canada)	PEL	Permissible Exposure Limits

EINECS	European Inventory of Existing 	RCRA	Resource Conservation and Recovery Act Chemical Substances

SARA	Superfund Amendments and Reauthorizations Act

EPA	Environmental Protection Agency	TLV	Threshold Limit Values

FDA	Food and Drug Administration	TQ	Threshold Quantities

HPA	Hazardous Products Act (Canada)	TWA	Time Weighted Average

IARC	International Agency for Research on Cancer	VOC	Volatile Organic Compounds

IATA	International Air Transport Association	WHMIS	Workplace Hazardous Materials Information System

This document is intended to provide a brief summary of our knowledge and guidance regarding the use of this material. The information set forth herein has been compiled from sources considered to be dependable and is believed to be accurate as of the date of publication. This information is offered in good faith by Encore Industrial and no warranty, expressed or implied is made. The user assumes all liability for any damage or injury resulting from misuse, from any failure to adhere to recommended practices or from any hazards inherent in the nature of the product.